Learning Report – Embedded C - Hardware + Programming + Testing

./

| Ver. Rel. No. | Release Date | Prepared. By | Reviewed By | Approved By | Remarks/Revision Details |
| --- | --- | --- | --- | --- | --- |
| 1.1 | 30-Sept-2020 | Anand A Warrier |  | Dr. Vivek Kaundal, Bhargav N | Initial draft report for review |

Document History

Contents

List of figures	4

Activity 1 – Writing linker scripts	5

Activity 2 – Semi Hosting technique	7

Activity 3 – In class activity	8

Activity 4 – Debugging techniques	10

Activity 5 – Completing MCU specific header files	15

# List of figures

Figure 1 - Linker script	5

Figure 2 - Semi hosting technique to print "Hello World"	6

Figure 3 - RCC bit enabled for SPI1 register	8

Figure 4 - SPE bit of SPI control register enabled	8

# Activity 1 – Writing linker scripts

Requirement: Create one memory location in the linker script.

Code:

//Entry point

ENTRY(Reset_Handler)

//Memories definition

MEMORY

{

FLASH(rx):ORIGIN =0x08000000,LENGTH =1024K

SRAM(rwx):ORIGIN =0x20000000,LENGTH =128K

}

//Sections

SECTIONS

{

.text :

{

*(.isr_vector)

*(.text)

*(.text.*)

*(.init)

*(.fini)

*(.rodata)

*(.rodata.*)

. = ALIGN(4);

_etext = .;

}> FLASH

_la_data = LOADADDR(.data);

.data :

{

_sdata = .;

*(.data)

*(.data.*)

. = ALIGN(4);

_edata = .;

}> SRAM AT> FLASH

.bss :

{

_sbss = .;

__bss_start__ = _sbss;

*(.bss)

*(.bss.*)

*(COMMON)

. = ALIGN(4);

_ebss = .;

__bss_end__ = _ebss;

. = ALIGN(4);

end = .;

__end__ = .;

}> SRAM

}	//close_Sections

Screenshot:

Figure 1 - Linker script

# Activity 2 – Semi Hosting technique

Requirement:

Code:

#include <stdio.h>

extern void initialise_monitor_handles(void);

int main(void)

{

initialise_monitor_handles();

printf("Hello World\n");

for(;;);

}

Screenshot:

Figure 2 - Semi hosting technique to print "Hello World"

# Activity 3 – In class activity

Requirement: Enable SPE bit position of SPI control register 1 (SPI_CR1) by writing embedded C code.

Code:

#include <stdint.h>

#define SPI_BASE_ADDR 		0x40013000UL

#define SPI_CR1_REG_OFFSET 	0x00UL

#define SPI_CR1_REG_ADDR 	(SPI_BASE_ADDR + SPI_CR1_REG_OFFSET)

#define RCC_BASE_ADDR				0x40023800UL

#define RCC_APB2ENR_REG_OFFSET		0x44UL

#define RCC_APB2ENR_REG_ADDR			(RCC_BASE_ADDR + RCC_APB2ENR_REG_OFFSET)

int main(void)

{

uint32_t *pSpiCr1Reg	=	(uint32_t*) SPI_CR1_REG_ADDR;

uint32_t *pRccApb2enrReg	=	(uint32_t*) RCC_APB2ENR_REG_ADDR;

*pRccApb2enrReg |= (1<<12); //enable the clock

*pSpiCr1Reg |= (1<<6);	//set the 6th bit i.e SPE bit

/* Loop forever */

for(;;);

}

Screenshot:

Figure 3 - RCC bit enabled for SPI1 register

Figure 4 - SPE bit of SPI control register enabled

# Activity 4 – Debugging techniques

Requirement: Take any C code and try these debugging options :

- Serial wire viewer and data tracing (printf style debugging)

- Single stepping, stepping over and stepping out.

- Breakpoints (inserting, deleting and skipping breakpoints).

- Disassembly

- Call stack

- Expression and variable window

- Memory browser

- Data watch points

Code:

main.c

#include <stdio.h>

int main(void)

{

printf("Hello World\n");

for(;;);

}

syscall.c

#include <sys/stat.h>

#include <stdlib.h>

#include <errno.h>

#include <stdio.h>

#include <signal.h>

#include <time.h>

#include <sys/time.h>

#include <sys/times.h>

/////////////////////////////////////////////////////////////////////////////////////////////////////////

//					Implementation of printf like feature using ARM Cortex M3/M4/ ITM functionality

//					This function will not work for ARM Cortex M0/M0+

//					If you are using Cortex M0, then you can use semihosting feature of openOCD

/////////////////////////////////////////////////////////////////////////////////////////////////////////

//Debug Exception and Monitor Control Register base address

#define DEMCR        			*((volatile uint32_t*) 0xE000EDFCU )

/* ITM register addresses */

#define ITM_STIMULUS_PORT0   	*((volatile uint32_t*) 0xE0000000 )

#define ITM_TRACE_EN          	*((volatile uint32_t*) 0xE0000E00 )

void ITM_SendChar(uint8_t ch)

{

//Enable TRCENA

DEMCR |= ( 1 << 24);

//enable stimulus port 0

ITM_TRACE_EN |= ( 1 << 0);

// read FIFO status in bit [0]:

while(!(ITM_STIMULUS_PORT0 & 1));

//Write to ITM stimulus port0

ITM_STIMULUS_PORT0 = ch;

}

/* Variables */

//#undef errno

extern int errno;

extern int __io_putchar(int ch) __attribute__((weak));

extern int __io_getchar(void) __attribute__((weak));

register char * stack_ptr asm("sp");

char *__env[1] = { 0 };

char **environ = __env;

/* Functions */

void initialise_monitor_handles()

{

}

int _getpid(void)

{

return 1;

}

int _kill(int pid, int sig)

{

errno = EINVAL;

return -1;

}

void _exit (int status)

{

_kill(status, -1);

while (1) {}		/* Make sure we hang here */

}

__attribute__((weak)) int _read(int file, char *ptr, int len)

{

int DataIdx;

for (DataIdx = 0; DataIdx < len; DataIdx++)

{

*ptr++ = __io_getchar();

}

return len;

}

__attribute__((weak)) int _write(int file, char *ptr, int len)

{

int DataIdx;

for (DataIdx = 0; DataIdx < len; DataIdx++)

{

//__io_putchar(*ptr++);

ITM_SendChar(*ptr++);

}

return len;

}

int _close(int file)

{

return -1;

}

int _fstat(int file, struct stat *st)

{

st->st_mode = S_IFCHR;

return 0;

}

int _isatty(int file)

{

return 1;

}

int _lseek(int file, int ptr, int dir)

{

return 0;

}

int _open(char *path, int flags, ...)

{

/* Pretend like we always fail */

return -1;

}

int _wait(int *status)

{

errno = ECHILD;

return -1;

}

int _unlink(char *name)

{

errno = ENOENT;

return -1;

}

int _times(struct tms *buf)

{

return -1;

}

int _stat(char *file, struct stat *st)

{

st->st_mode = S_IFCHR;

return 0;

}

int _link(char *old, char *new)

{

errno = EMLINK;

return -1;

}

int _fork(void)

{

errno = EAGAIN;

return -1;

}

int _execve(char *name, char **argv, char **env)

{

errno = ENOMEM;

return -1;

}

Screenshot:

Figure 5 - Serial Wire Viewer

Figure 6 - Variable window

# Activity 5 – Completing MCU specific header files

Requirement: